SIMATS ENGINEERING

ASSESSMENT TOOL 1

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques

using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 40%

Annexure A

ANALYTICAL PROBLEM SOLVING

1.A university network is assigned the IP address block 192.168.20.0/24 for its internal communication. The network administrator has decided to divide this network into subnets for three different labs using fixed subnet masks. Lab A is assigned a /26 subnet, Lab B is assigned a /27 subnet, and Lab C is assigned a /28 subnet. Based on the given subnet masks, apply sub-netting concepts to determine the network address of each lab. Further, identify the valid range of host IP addresses and the broadcast address for each subnet. Finally, calculate the total number of usable host IP addresses available in each lab’s subnet.

SOLUTION:

| Lab | Subnet Mask | Network Address | Valid Host Range | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| Lab A | /26 (255.255.255.192) | 192.168.20.0 | 192.168.20.1 – 192.168.20.62 | 192.168.20.63 | 62 |
| Lab B | /27 (255.255.255.224) | 192.168.20.64 | 192.168.20.65 – 192.168.20.94 | 192.168.20.95 | 30 |
| Lab C | /28 (255.255.255.240) | 192.168.20.96 | 192.168.20.97 – 192.168.20.110 | 192.168.20.111 | 14 |

Lab A (/26)

- Total addresses = 2^(32−26) = 64

- Usable hosts = 64 − 2 = 62

Lab B (/27)

- Total addresses = 32

- Usable hosts = 32 − 2 = 30

Lab C (/28)

- Total addresses = 16

- Usable hosts = 16 − 2 = 14

Final Answer

- Lab A → Network: 192.168.20.0/26, Broadcast: 192.168.20.63, Hosts: 62

- Lab B → Network: 192.168.20.64/27, Broadcast: 192.168.20.95, Hosts: 30

- Lab C → Network: 192.168.20.96/28, Broadcast: 192.168.20.111, Hosts: 14

2.  A network administrator has configured two systems with the following details: System 1 has IP address 192.168.1.130 with subnet mask 255.255.255.192, and System 2 has IP address 192.168.1.190 with subnet mask 255.255.255.192. Using the given configuration, apply subnetting techniques to determine the subnet range, network address, and broadcast address for each system. Identify the valid host IP range for both subnets based on the given subnet mask. Calculate whether each IP address falls within its respective subnet range. Also, determine the number of usable host addresses available in each subnet.

SOLUTION:

### Given

- System 1: 192.168.1.130

- System 2: 192.168.1.190

- Subnet Mask: 255.255.255.192 (/26)

A /26 subnet has a block size of 64.

Subnets are:

- 192.168.1.0 – 63

- 192.168.1.64 – 127

- 192.168.1.128 – 191

- 192.168.1.192 – 255

### System 1

IP Address = 192.168.1.130

| Item | Value |
| --- | --- |
| Network Address | 192.168.1.128 |
| Valid Host Range | 192.168.1.129 – 192.168.1.190 |
| Broadcast Address | 192.168.1.191 |
| Usable Hosts | 62 |

Verification

192.168.1.130 lies between 129 and 190.

Therefore, it belongs to this subnet.

### System 2

IP Address = 192.168.1.190

| Item | Value |
| --- | --- |
| Network Address | 192.168.1.128 |
| Valid Host Range | 192.168.1.129 – 192.168.1.190 |
| Broadcast Address | 192.168.1.191 |
| Usable Hosts | 62 |

Verification

192.168.1.190 is the last valid host.

Therefore, it belongs to this subnet

3. An organization has been allocated the IP block 172.16.0.0/16 for its internal network. The network administrator has already decided the subnet masks for each department as follows: HR → /25, Sales → /26, IT → /27, and Admin → /28. Using the given subnet masks, apply subnetting techniques to assign suitable subnet addresses for each department. Determine the network address, valid host IP range, and broadcast address for each subnet. Based on the allocation, calculate the number of usable host addresses in each department. Finally, identify the remaining unused IP address range within the given network block.

SOLUTION:

Given Network: 172.16.0.0/16

| Department | Prefix | Network Address | Host Range | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| HR | /25 | 172.16.0.0 | 172.16.0.1 – 172.16.0.126 | 172.16.0.127 | 126 |
| Sales | /26 | 172.16.0.128 | 172.16.0.129 – 172.16.0.190 | 172.16.0.191 | 62 |
| IT | /27 | 172.16.0.192 | 172.16.0.193 – 172.16.0.222 | 172.16.0.223 | 30 |
| Admin | /28 | 172.16.0.224 | 172.16.0.225 – 172.16.0.238 | 172.16.0.239 | 14 |

### Host Calculation

- HR → 128 − 2 = 126

- Sales → 64 − 2 = 62

- IT → 32 − 2 = 30

- Admin → 16 − 2 = 14

### Remaining Unused IP Range

Allocated till: 172.16.0.239

Remaining:  172.16.0.240 – 172.16.255.255

4. A company uses the IP address 172.16.10.0/24 and has implemented subnetting using a /27 subnet mask for its internal departments. Using the given subnet mask, apply subnetting techniques to calculate the total number of subnets created and the number of usable hosts per subnet. Determine the subnet to which the IP address 172.16.10.78 belongs. Identify the network address and broadcast address for that subnet. Also, determine whether the IP address 172.16.10.95 falls within the same subnet range based on the given configuration.

SOLUTION:

Given:

- Network = 172.16.10.0/24

- Subnet Mask = /27

### (a) Number of Subnets

Borrowed bits:

27 − 24 = 3

Number of subnets:

2³ = 8

### (b) Usable Hosts

Hosts:

2^(32−27) = 32

Usable:

32 − 2 = 30 hosts

### (c) Subnets

| Subnet | Range |
| --- | --- |
| 172.16.10.0/27 | 0–31 |
| 172.16.10.32/27 | 32–63 |
| 172.16.10.64/27 | 64–95 |
| 172.16.10.96/27 | 96–127 |
| 172.16.10.128/27 | 128–159 |
| 172.16.10.160/27 | 160–191 |
| 172.16.10.192/27 | 192–223 |
| 172.16.10.224/27 | 224–255 |

### (d) IP Address 172.16.10.78

78 falls between 64–95

Network Address: 172.16.10.64

Broadcast Address: 172.16.10.95

Host Range: 172.16.10.65 – 172.16.10.94

### (e) Does 172.16.10.95 belong to same subnet?

Yes.

172.16.10.95 is the broadcast address of the same subnet.

It belongs to the subnet but cannot be assigned to a host.

5. A network uses the Internet Protocol version 6 address 2001:0db8:0000:0000:0000:ff00:0042:8329. Apply IPv6 addressing rules to compress the given address into its shortest form and then expand the compressed address back to its full notation. Using a prefix length of /64, determine the network prefix of the given address. Also, calculate the total number of possible host addresses available within this network based on the given prefix length.

SOLUTION:

Given IPv6 Address:

2001:0db8:0000:0000:0000:ff00:0042:8329

## (a) Compressed Form

Remove leading zeros and replace consecutive zero groups with "::"

2001:db8::ff00:42:8329

## (b) Expanded Form

2001:0db8:0000:0000:0000:ff00:0042:8329

## (c) Network Prefix (/64)

First 64 bits (first four groups):

2001:0db8:0000:0000::/64

or

2001:db8:0:0::/64

## (d) Total Host Addresses

Host bits:

128 − 64 = 64 bits

Total host addresses:

2⁶⁴ = 18,446,744,073,709,551,616