SIMATS ENGINEERING

ASSESSMENT TOOL 2

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques

using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Annexure A

Scenario-Based

1. A newly established Smart Campus is deploying a secure network

infrastructure for its academic and administrative departments. The network

administrator has been allocated the IP address block 192.168.100.0/24. Based

on the expected number of devices, the following subnet masks have already

been assigned:

School of Computing → /25

School of Electronics → /26

School of Mechanical Engineering → /27

As the network engineer, design the subnet allocation by performing the

following tasks:

1. Determine the network address and broadcast address for each

department.

2. Identify the valid host IP address range for each subnet.

3. Calculate the number of usable host addresses available in each subnet.

4. Determine the remaining unused IP address range within the given

network block.

SOLUTION:

| Department | Subnet Mask | Network Address | Valid Host Range | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| School of Computing | /25 | 192.168.100.0 | 192.168.100.1 – 192.168.100.126 | 192.168.100.127 | 126 |
| School of Electronics | /26 | 192.168.100.128 | 192.168.100.129 – 192.168.100.190 | 192.168.100.191 | 62 |
| School of Mechanical Engineering | /27 | 192.168.100.192 | 192.168.100.193 – 192.168.100.222 | 192.168.100.223 | 30 |

### Remaining Unused IP Address Range

- 192.168.100.224 – 192.168.100.255

- Equivalent subnet: 192.168.100.224/27

- Total remaining addresses = 32 (30 usable hosts)

2. A multinational software company is expanding its headquarters and has been

assigned the private IP block 10.10.0.0/16. Different departments require

different subnet sizes based on the number of employees. The network

administrator has allocated the following subnet masks:

Software Development → /24

Quality Assurance → /25

Human Resources → /26

Executive Management → /27

As the network administrator, perform the following tasks:

1. Determine the network address and broadcast address for each

department.

2. Identify the valid range of host IP addresses.

3. Calculate the number of usable hosts in each subnet.

4. Identify the unused IP address range remaining within the allocated IP

block.

SOLUTION:

| Department | Subnet Mask | Network Address | Valid Host Range | Broadcast Address | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| Software Development | /24 | 10.10.0.0 | 10.10.0.1 – 10.10.0.254 | 10.10.0.255 | 254 |
| Quality Assurance | /25 | 10.10.1.0 | 10.10.1.1 – 10.10.1.126 | 10.10.1.127 | 126 |
| Human Resources | /26 | 10.10.1.128 | 10.10.1.129 – 10.10.1.190 | 10.10.1.191 | 62 |
| Executive Management | /27 | 10.10.1.192 | 10.10.1.193 – 10.10.1.222 | 10.10.1.223 | 30 |

### Remaining Unused IP Address Range:

Remaining addresses within the allocated /16 block begin from:

- 10.10.1.224 – 10.10.255.255

(These addresses remain available for future subnet allocation.)

3. A smart hospital is migrating its network infrastructure from IPv4 to IPv6 to

support thousands of connected medical devices and IoT sensors. The hospital

has been assigned the IPv6 network:

2001:0db8:abcd:0000:0000:0000:0000:0000/64

As the network engineer responsible for the migration, perform the following:

1. Convert the given IPv6 address into its compressed notation.

2. Expand the compressed address back into its full hexadecimal

representation.

3. Identify the network prefix using the /64 prefix length.

4. Calculate the total number of host addresses supported within the

subnet.

5. the total number of possible host addresses available within this network.

SOLUTION:

Given IPv6 Address

2001:0db8:abcd:0000:0000:0000:0000:0000/64

### 1. Compressed Notation

2001:db8:abcd::/64

### 2. Expanded Form

2001:0db8:abcd:0000:0000:0000:0000:0000/64

### 3. Network Prefix (/64)

Network Prefix: 2001:db8:abcd:0000::/64

or simply

2001:db8:abcd::/64

### 4. Total Number of Host Addresses Supported

Host bits = 128 − 64 = 64

Total addresses

= 2⁶⁴

= 18,446,744,073,709,551,616 addresses

### 5. Total Possible Host Addresses

2⁶⁴ = 18,446,744,073,709,551,616

4. An Internet Service Provider (ISP) connects multiple customer networks

through a core router. The router maintains the following routing table entries:

192.168.1.0/24

192.168.2.0/24

192.168.3.0/24 A packet arrives at the router with the destination IP address:

192.168.2.45

As the routing engineer, determine:

1. The destination subnet to which the packet belongs.

2. The matching routing table entry.

3. The next-hop network/interface used to forward the packet.

4. The default route that should be used if the destination does not match

any routing table entry.

SOLUTION:

### Routing Table

- 192.168.1.0/24

- 192.168.2.0/24

- 192.168.3.0/24

Destination IP: 192.168.2.45

### 1. Destination Subnet

192.168.2.0/24

### 2. Matching Routing Table Entry

192.168.2.0/24

### 3. Next-Hop Network / Interface

Forward the packet through the interface connected to:

192.168.2.0/24 network

### 4. Default Route

If no route matches:

0.0.0.0/0

(Default Gateway)

5. A company establishes two office departments connected through a Layer-3

router. The allocated subnet information is as follows:

Administration LAN → 192.168.10.0/26

Finance LAN → 192.168.10.64/26

The router interfaces are configured with:

Administration Gateway → 192.168.10.1

Finance Gateway → 192.168.10.65

As the network administrator, perform the following tasks:

1. Determine the network address and broadcast address for each LAN.

2. Identify the valid host IP address range.

3. Suggest suitable IP addresses for three computers in each LAN.

4. Identify the default gateway that hosts in each LAN should use for

communication.

SOLUTION:

## Administration LAN

Network: 192.168.10.0/26

- Network Address: 192.168.10.0

- Broadcast Address: 192.168.10.63

- Valid Hosts:

  - 192.168.10.1 – 192.168.10.62

- Default Gateway:

  - 192.168.10.1

### Suggested IP Addresses

- PC1 → 192.168.10.2

- PC2 → 192.168.10.3

- PC3 → 192.168.10.4

## Finance LAN

Network: 192.168.10.64/26

- Network Address: 192.168.10.64

- Broadcast Address: 192.168.10.127

- Valid Hosts:

  - 192.168.10.65 – 192.168.10.126

- Default Gateway:

  - 192.168.10.65

### Suggested IP Addresses

- PC1 → 192.168.10.66

- PC2 → 192.168.10.67

- PC3 → 192.168.10.68