SIMATS ENGINEERING

ASSESSMENT TOOL 3

COURSE NAME: COMPUTER NETWORKS

COURSE CODE: CSA07

COURSE OUTCOME COVERED

CO2: Configure IP addressing schemes, subnetting, and basic routing techniques

using simulation tool, for efficient network design and topology. (BL3)

Assessment Tool Weightage: 30%

Software:Cisco Packet Tracer,Wireshark

Annexure A

Debugging &optimisation Task

1. A network administrator has configured a workstation with the following settings:

 IP Address: 192.168.10.65

 Subnet Mask: 255.255.255.192 (/26)

 Default Gateway: 192.168.10.1

However, the workstation is unable to communicate with other systems configured in the subnet: 192.168.10.0/26.

Task

Debug the network configuration by:

 Identifying the subnet to which the configured IP address belongs.

 Determining the network address, broadcast address, and valid host range.

 Verifying whether the configured IP address and gateway are valid.

 Identifying the configuration errors.

 Optimizing the configuration by assigning an appropriate IP address and default gateway.

 Calculating the total number of usable host addresses after correction.

SOLUTION:

Given:

- IP Address: 192.168.10.65

- Subnet Mask: 255.255.255.192 (/26)

- Default Gateway: 192.168.10.1

| Parameter | Answer |
| --- | --- |
| Subnet | 192.168.10.64/26 |
| Network Address | 192.168.10.64 |
| Broadcast Address | 192.168.10.127 |
| Valid Host Range | 192.168.10.65 – 192.168.10.126 |
| IP Address Validity | Valid |
| Gateway Validity | Invalid (belongs to a different subnet) |
| Configuration Error | Default gateway is not in the same subnet as the workstation. |
| Optimized Configuration | IP: 192.168.10.65, Gateway: 192.168.10.126 |
| Usable Host Addresses | 62 |

2. Two devices are configured with the following IPv6 addresses:

 Device A: 2001:db8::ff00:42:8329/64

 Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

Task

Debug the IPv6 configuration by:

 Expanding both IPv6 addresses into their complete notation.

 Determining the network prefix of each device.

 Verifying whether both devices belong to the same subnet.

 Identifying the addressing or prefix mismatch.

 Optimizing the IPv6 configuration by assigning appropriate addresses.

 Calculating the number of available host addresses within the corrected subnet.

SOLUTION:

Given:

- Device A: 2001:db8::ff00:42:8329/64

- Device B: 2001:db8:0:0:1::1/64

| Parameter | Answer |
| --- | --- |
| Expanded Address (A) | 2001:0db8:0000:0000:0000:ff00:0042:8329 |
| Expanded Address (B) | 2001:0db8:0000:0000:0001:0000:0000:0001 |
| Network Prefix (A) | 2001:db8:0:0::/64 |
| Network Prefix (B) | 2001:db8:0:0::/64 |
| Same Subnet | Yes |
| Addressing Issue | No prefix mismatch; communication failure is likely due to another network configuration issue. |
| Optimized Addresses | Device A: 2001:db8:0:0::10/64 Device B: 2001:db8:0:0::20/64 |
| Available Host Addresses | 2⁶⁴ (18,446,744,073,709,551,616) |

3. An organization has allocated the network 192.168.1.0/24 into several departments using the

subnet masks /26, /27, and /28. Some departments report insufficient IP addresses, while others

have a large number of unused addresses.

Task

Analyze and optimize the subnet allocation by:

 Identifying the network address, broadcast address, and host range for each subnet.

 Calculating the number of usable and unused IP addresses.

 Identifying inefficient subnet allocations.

 Debugging the addressing plan by locating subnet allocation mistakes.

 Recommending an optimized subnetting scheme using VLSM to improve address

utilization while supporting current departmental requirements.

SOLUTION:

Network: 192.168.1.0/24

| Subnet | Network Address | Broadcast Address | Host Range | Usable Hosts |
| --- | --- | --- | --- | --- |
| /26 | 192.168.1.0 | 192.168.1.63 | 192.168.1.1 – 192.168.1.62 | 62 |
| /27 | 192.168.1.64 | 192.168.1.95 | 192.168.1.65 – 192.168.1.94 | 30 |

  Unused IP Addresses: 144 (192.168.1.112 – 192.168.1.255)

  Inefficient Allocation: Larger subnets may waste addresses while smaller departments may face shortages.

  Optimized Scheme: Use VLSM, assigning larger subnets only where needed and smaller subnets to departments with fewer hosts to improve address utilization.

4. Two devices are configured with the following IPv6 addresses:

 Device A: 2001:db8::ff00:42:8329/64

 Device B: 2001:db8:0:0:1::1/64

Although both devices are connected to the same network, communication fails.

Task

Debug the IPv6 configuration by:

 Expanding both IPv6 addresses into their complete notation.

 Determining the network prefix of each device.

 Verifying whether both devices belong to the same subnet.

 Identifying the addressing or prefix mismatch.

 Optimizing the IPv6 configuration by assigning appropriate addresses.

 Calculating the number of available host addresses within the corrected subnet.

SOLUTION:

Given:

- Device A: 2001:db8::ff00:42:8329/64

- Device B: 2001:db8:0:0:1::1/64

| Parameter | Answer |
| --- | --- |
| Expanded Address (A) | 2001:0db8:0000:0000:0000:ff00:0042:8329 |
| Expanded Address (B) | 2001:0db8:0000:0000:0001:0000:0000:0001 |
| Network Prefix (A) | 2001:db8:0:0::/64 |
| Network Prefix (B) | 2001:db8:0:0::/64 |
| Same Subnet | Yes |
| Addressing Issue | No prefix mismatch; communication failure is likely due to another network configuration issue. |
| Optimized Addresses | Device A: 2001:db8:0:0::10/64 Device B: 2001:db8:0:0::20/64 |
| Available Host Addresses | 2⁶⁴ (18,446,744,073,709,551,616) |

5. A router contains the following routing table.

Destination Network Next Hop

192.168.1.0/24 10.0.0.2

192.168.2.0/24 10.0.0.3

Default Route 10.0.0.1

Users report that packets destined for 192.168.2.100 are not reaching the intended network.

Task

Debug the routing configuration by:

 Identifying the matching routing entry using the longest-prefix match.

 Determining the next-hop router selected for packet forwarding.

 Verifying whether the destination IP belongs to the intended subnet.

 Identifying any routing configuration errors.

 Optimizing the routing table to improve packet forwarding efficiency.

 Identifying the route that will be used if no matching destination exists.

SOLUTION:

- Routing Table

| Destination Network | Next Hop |
| --- | --- |
| 192.168.1.0/24 | 10.0.0.2 |
| 192.168.2.0/24 | 10.0.0.3 |
| Default Route | 10.0.0.1 |

- Destination IP: 192.168.2.100

| Parameter | Answer |
| --- | --- |
| Matching Route | 192.168.2.0/24 |
| Next Hop | 10.0.0.3 |
| Destination Subnet Verification | Destination belongs to 192.168.2.0/24 |
| Routing Error | No routing entry error; communication failure may be due to an unreachable next hop or interface issue. |
| Optimized Routing | Verify next-hop connectivity and interface status; use route summarization if appropriate. |
| Default Route Used | 10.0.0.1 (when no specific route matches) |